# Approximate Muon Yields

0.30 + per incident proton @ 2.2 GeV (Ta target)

Optimised yield at end of 30 m solenoid decay channel:

0.95% (120-260 MeV) + per (proton.GeV)

Approximate yield after chicane and 190-400 MeV linac:

(assuming hard-edged magnets and solenoids in Parmila)

0.85% + per (proton.GeV)

Simulations with Muonplayer code now commencing

Two chicane (B1, B2) magnet lattices are now under study

Lower yields expected due to chicane magnet end fields

Yields down by 50% still equivalent to  the CERN scheme