Change Request

| Project | Change Request No. |
| --- | --- |
| MAPS for CALICE | 1 |

Part A (to be completed by the Originator)

| Description of Change Requested |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Investigate whether additional resistance can be added to preShape pixel feedback resistor. Suspected error in pcells means the original design value of 4Mohms was really 2.7Mohms. Target is to lengthen the feedback resistor so its value is closer to 4Mohm but with sensible layout (ie guard rings, not next to digital logic etc). Will be reviewed at FDR and approved for use or rejected, in which case the original design value (2.7Mohms) will be made instead. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Anticipated Benefits/Reason for Change |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Improvement to signal gain in preShape pixel (towards that originally designed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Please click on the box with the most likely result of the change in each row |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Substantial Reduction |  |  | Reduction |  |  | No change |  |  |  | Increase |  |  |  | Substantial Increase |  |
| Cost |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Schedule |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Performance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Resources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minor effort required ~1 day to make new layout as drop-in replacement for current (new) pixel design. Minor risk associated with modification to analog front-end circuits. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Originator |  |  |  |  |  | JC |  |  |  |  |  |  |  |  |  |  |
| Signature |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |
| Approval Required in (click on box) |  | 1 week |  |  | 2 weeks |  |  |  | 1 month |  |  |  |  | 2 months |  |  |

Part B (to be completed by the Project Manager)

| Recommended for (click on box) |  | Project Manager Signature | Date |
| --- | --- | --- | --- |
| Implementation  | Rejection |  |  |
| Approval for Implementation |  | Customer/Sponsor Signature | Date |
| JC |  |  |  |
| Action Required/Comments |  |  |  |
| Layout value of 3.3Mohm is feasible. Simulation of circuit with new values will be presented at FDR with layout for approval. |  |  |  |