Tera-Pixel APS for CALICE

Technical Specification

Document Revision 0.1 (DRAFT)

Jamie Crooks

29 July 2026

|  | Name | Signature | Date |
| --- | --- | --- | --- |
| Project Manager | Jamie Crooks |  |  |
| Customer/Sponsor | Paul Dauncey |  |  |

# Table of Contents

1.	Table of Contents	2

2.	Table of initial requirements	3

2.1	ILC accelerator worst case assumptions	3

2.2	MAPS Requirements	3

3.	Overview timetable	5

4.	Specification of ASIC1 & ASIC2	6

4.1	General	6

4.2	System	7

4.3	Pixel	8

4.4	Requirements that may/will not be met	10

4.4.1	Memories	10

4.4.2	System / Global	10

4.4.3	Functional/Pixel	11

5.	ASIC1 Guide Floorplan	12

6.	ASIC2 Guide Floorplan	12

# Table of initial requirements

This section contains a tabulated version of the initial requirements document provided at the very start of the feasibility study.  These requirements form a good introduction to the overall project goals.

## ILC accelerator worst case assumptions

| Beam crossing period | 150ns, so rate = 6.7MHz |
| --- | --- |
| Number of crossings per train | 14000, so length = 2ms |
| Train repetition rate | 10Hz, so period = 100ms , duty factor = 2% |
| Luminosity | 5x10^34/cm^2/s, integrated lumi per train = 5/nb |

## MAPS Requirements

| Pixel response | The pixels must give one hit per charged particle crossing the sensor. This is of course impossible in practise but is the ideal we should aim to be close to |
| --- | --- |
| Maximum inefficiency (defined as one particle giving no hit) | 5% within pixel (also see below). |
| Maximum crosstalk (defined as one particle giving two hits) | 5% (?) within pixel. |
| Maximum noise hit rate | 10^-5, target rate 10^-6. The noise rate must be insensitive to expected temperature changes (tbd). |
| Maximum dead space | 10%. Working assumption is that basic sensor structure is 2x2cm^2 with a 250mu contact pad strip around the outside. This gives 5% dead space; the other 5% is random dead areas over the pixels (bad pixels, non-sensitive n-well areas, etc), as above. |
| Pixel size | minimum size 20x20mu^2 maximum size 60x60mu^2. |
| Comparator | The comparator/storage cycle time must occur within 100ns of a trigger signal. |
|  | The trigger should be able to come a minimum of 50ns after the particles cross the sensor |
|  | The comparator threshold must be set/trimmed by on-sensor DAC. The DAC should be common to multiple pixels, which are therefore required to be uniform |
|  | The threshold setting must be insensitive to expected temperature changes |
| Working assumptions | Triggers will have minimum spacing of 150ns but could be arbitrary times apart (many seconds for cosmics). |
|  | Worsening the time resolution to more than 150ns could be done if necessary but is not very desirable. |
| Memory | The timestamps can require up to 16 bits |
|  | The memory needs to be able to hold data for at least hundreds of ms for ILC operation. For beam tests, sources and cosmics, it would be useful to have the memory hold data for many seconds. |
|  | All data in memories must be able to be read out within 98ms under normal conditions (reasonable occupancy, etc, tbd). |
|  | Bad pixels should be able to be masked |
|  | Pixels with no hits should be ignored during readout |
| The number of allowed timestamps | Minimum = 4, Maximum number = 16. |
| External contacts | The contacts should mechanically allow the sensor to be placed inverted on a PCB preferably using standard BGA-like solder technology |
|  | All required I/O termination circuitry should be within the sensor |
|  | No substrate ground should be needed |
|  | Data output must be able to drive signals down 1.5m tracks on a PCB |
| Power | The maximum averaged power during timing equivalent to ILC operation must be 1 uW/mm^2 |

# Overview timetable

For a more detailed project plan please refer to the Gantt chart accompanying the Project Management Plan document.

# Specification of ASIC1 & ASIC2

It is important to identify the key requirements that will be addressed in the first and second ASIC designs.  In general, the first ASIC will be primarily concerned with the pixel design, and the second ASIC will develop this into a larger device considering system and global requirements.

## General

| ASIC1 | ASIC2 |
| --- | --- |
| Test Structures: Opportunity to explore several different designs depending on design study. Specific pixel variants will become clear during the design process, and will not be defined in this specification. The preferred approach would be maximum 4 pixel variants forming one active array with common control signals (see example floor plan in section 5) | Single large design: Selecting specific sub-designs from ASIC1 based on test results. |
| Sub-reticle size, possibly MPW run if available / cost efficient. | Full reticle or stitched design, engineering run to produce enough sensors for final beam test |
| External components will be used | Use of external components will be minimised |
| No constraint on number of control and power signals (pads) | Number of control and power signals will be minimised |
| No constraint on overall power consumption, but circuits will be selected for long-term low power goal. | Total averaged power consumption will be minimised |
| Pads located at chip edge for wire bonding. Pad cells designed to meet bump-bonding specification for assembly tests. | Will consider locating pads on one side or in one central region (depending on success of bump bonding trials and limitations of stitching process) |
| Technology: to be selected - Foundry A (0.35um) - Foundry B (0.18um) | Technology will not change for ASIC2, allowing circuits designed and proven in ASIC1 to be reused with confidence. |

## System

| ASIC1 | ASIC2 |
| --- | --- |
| Will aim to maximise active area in pixel regions only | Will aim to maximise active area across full ASIC area |
| Parallel data output for debug | Will consider high-speed serial data output, eg LVDS. |
| No data reduction on-chip | Will consider data reduction techniques |
| No peripheral data storage, live parallel data must be captured or lost | Will consider peripheral data storage (FIFO) to support possible handshaking/request protocol from controller FPGA/system. |
| Sparse pixel readout should be demonstrated in this device (using mostly external control signals) | Sparse readout should be incorporated into on-chip control logic based on evaluation of first device. |
| Comparator threshold is driven from off-chip. If identified as a critical circuit, a trial DAC could be implemented in ASIC1 for evaluation independent of sensor operation. | Comparator threshold is generated on-chip with digital programmable DAC. |

## Pixel

| ASIC1 | ASIC2 |
| --- | --- |
| 50 microns pixel size | 50 microns pixel size |
| EPI thickness: Maximum offered on chosen technology: >12um. | No change |
| 4 diodes per pixel Physics simulations will define precise location of each. Could explore the single-diode case as a pixel variant (noise-rate / crosstalk trade-off) – or different analog-sum circuits, subject to physics & circuit simulations | 4 diodes per pixel (subject to ASIC1 testing) Physics simulations may be required to redefine precise location of each if in-pixel circuits change between ASIC1 and ASIC2. |
| Diode type: to be selected from available technology options based on physics simulations. Eg: Standard/Deep NWELL? Different diode structures could be implemented as pixel variants for comparative testing, depending on physics simulations | Diode type will be selected based on physics testing of ASIC1 |
| Diode reset: possible options include continuous / Pulsed reset Different diode circuits could be implemented as pixel variants for comparative testing | Diode reset: to be selected based on ASIC1 testing |
| Minimum detectable input signals 1 MIP |  |
| “Fire” threshold ~50% equivalent MIP signal, subject to physics simulations |  |
| Target noise rate (false hits): 10^5 | Target noise rate: 10^6 |

| ASIC1 | ASIC2 |
| --- | --- |
| Comparator design study to identify a suitable comparator design to meet the above requirements: Either local threshold trimming or autozero will be necessary to overcome mismatch, reset noise (if applicable) to meet target noise rate. Clocked/continuous circuits will be considered and evaluated for their low-power operation. Different comparator circuits could be implemented as pixel variants for comparative testing, if the design study identifies more than one suitable candidate. | Comparator design to be selected based on ASIC1 testing. Possible redesign if measured noise rate will fail to meet the target above. |
| 4 in-pixel memories | No change, unless ASIC1 test results cause reconsideration. |
| DRAM Technology is selected, since no pmos devices are used improving pixel charge collection. |  |
| Working assumption ~5 ms DRAM lifetime sets readout window (<<98ms available!) |  |
| 16 bit pixel memories | 16 bit pixel memories |
| Outcome from feasibility study recommends implementing the memory management logic for a set of pixels in one dead pixel called a “local controller”. This reduces the logic in the pixel (improving sensitivity) at the cost of one known dead pixel per 2N active pixels & high routing density (negligible impact for high energy particles). Depending on process technology chosen, N might range from 8 to 15, yielding an average dead space of 6% to 3% respectively. |  |

## Requirements that may/will not be met

The specifications listed above do not necessarily meet the full set of initial requirements set out in section 2, but have evolved over the course of the feasibility study and regular monthly meetings with the collaborators.  This section highlights those initial requirements that will not be met, or have had to be compromised in this development plan.

### Memories

| The number of allowed timestamps | Minimum = 4, Maximum number = 16. |
| --- | --- |

Due to space limitations in the pixel, the number of registers is very unlikely to increase over the minimum (4).  More registers increases pixel complexity, routing & control logic which have a detrimental effect on dead space.

| Memory | The memory needs to be able to hold data for at least hundreds of ms for ILC operation. For beam tests, sources and cosmics, it would be useful to have the memory hold data for many seconds. |
| --- | --- |

Since DRAM circuits are targeted for this design there exists a finite data-retention time of order 5ms with no refresh cycle.  Refresh cycles are not intended to be implemented, since event and noise hits from a bunch train are calculated to be feasible to read out within the 5ms.

This impacts the applications cited as requiring “hundreds of ms” data hold time which should be re-considered.

### System / Global

| External contacts | All required I/O termination circuitry should be within the sensor |
| --- | --- |

Where possible termination and decoupling will be included on-chip for ASIC2.  Achieving zero external components may prove difficult, but will be considered during ASIC2 development: A likely outcome is an ASIC2 that could be evaluated with/without external components for performance comparisons.

| Power | The maximum averaged power during timing equivalent to ILC operation must be 1 uW/mm^2 |
| --- | --- |

This specification has already been identified as extremely tight, and is unlikely to be met in either ASIC1 or ASIC2.  The focus of ASIC2 will be low power operation, so whilst the target is unobtainable, the sensor produced should demonstrate the realistic extent of the low-power capabilities of current technology.

### Functional/Pixel

| Comparator | The comparator threshold must be set/trimmed by on-sensor DAC. The DAC should be common to multiple pixels, which are therefore required to be uniform |
| --- | --- |

The exact implementation of the comparator is not yet known.  The issue of non-uniformity between pixels will be addressed throughout (primarily ASIC1) but the specific approach first identified may not be the winning solution.  Local DAC trimming may prove to be costly solution, where auto-zero comparator designs may be more appropriate.  An initial design study will investigate and draw conclusions, with the overall final choice brought back to the collaboration meeting.

# ASIC1 Guide Floorplan



64x64



64x64



64x64



64x64

10mm

10mm

Readout

Control

Pad & Power Ring

Pixels

Four pixel variants with shared control and readout circuits so the device behaves as a single sensor.

Overall active area

= 128x128 pixels @ 50um

=  6.4mm x 6.4mm

30-60mm (TBD)

30-60mm (TBD)



600x600?

1200x1200?

ASIC2 Guide Floorplan

Single pixel variant is selected.

Large sensitive area (possibly stitched) with optimised periphery circuits