Currents

|  | Name |  | Locate | Specification | Comment | DAC setting (dec.) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | I34_IOUTBIAS | npn | R65 gnd | n/a | PCB mod, direct 100ohm to ground |  |
| 1 | I34_MSOBIAS2 | npn | R67 gnd | 90uA out of pad | Change R67 to 5K1 |  |
| 2 | I34_MSOBIAS1 | npn | R66 gnd | 90uA out of pad | Change R66 to 5K1 |  |
| 3 | I34_COMPBIAS2 | npn | R68 gnd | 100uA out of pad | Change R68 to 560R | 830 |
| 4 | I34_PREBIAS | npn | R70 gnd | 130uA out of pad | Change R70 to 560R | 1030 |
| 5 | I34_OUTSFBIAS | npn | R71 gnd | 120uA out of pad | Change R71 to 560R | 630 |
| 6 | I34_COMPBIAS1 | pnp | R72  vdd | 65uA into pad | Change R72 to 20K | 3100 |
| 7 | I34_SFBIAS | pnp | R69  vdd | 90uA into pad | Change R69 to 13K | 2450 |
| 8 | I34_COMPBIASTRIM | pnp | R73  vdd |  | Change R73 to 100K | 3000 |
| 9 | DEBUGSFBIAS | pnp | R74  vdd | 200uA into pad | Change R74 to 6K2 | (~3000 TBC) |
| 10 | ISENSE_COLREF | pnp | R76  vdd |  |  |  |
| 11 | I12COMPBIASTRIM | pnp | R75  vdd |  | Change R75 to 100K | 3000 |
| 12 | I12SHAPERBIAS | npn | R79 gnd | 150uA out of pad | Change R79 to 560R | 290 * |
| 13 | I12_PREBIAS | npn | R78 gnd | 150uA out of pad | Change R78 to 560R | 290 * |
| 14 | I12_COMPBIAS2 | npn | R80 gnd | 83uA out of pad | Change R80 to 560R | 830 |
| 15 | I12_MSOBIAS1 | npn | R77 gnd | 90uA out of pad | Change R77 to 5K1 |  |
| 16 | I12_OUTBIAS | npn | R85  gnd | n/a | PCB mod, direct 100ohm to ground |  |
| 17 | ISENSE_ICOMPBIAS | npn | R82  gnd |  |  |  |
| 18 | ISENSE_IOUTBIAS | npn | R81  gnd |  |  |  |
| 19 | I12_COMPBIAS1 | pnp | R83  vdd | 65uA into pad | Change R83 to 20K | 3100 |
| 20 | ISENSEBIAS | pnp | R84  vdd |  | Change R84 to 110R |  |

* below spec

Voltages

|  | Name |  |  | Specification | Comment | DAC setting (dec.) |
| --- | --- | --- | --- | --- | --- | --- |
| 21 | VTH-34 |  |  |  |  |  |
| 22 | DEBUG_VTH- |  |  |  |  |  |
| 23 | VTH-12 |  |  |  |  |  |
| 24 | VRST |  |  | 1.0v |  | 2275 |
| 25 | VTH+34 |  |  |  | Should be greater than VTH-34 |  |
| 26 | VPREAMPCASC34 |  |  | 1.5v |  | 3400 |
| 27 | DEBUG_VTH+ |  |  |  | Should be greater than DEBUG_VTH- |  |
| 28 | VSHAPECASC12 |  |  | 1.5v |  | 3400 |
| 29 | VPRECASC12 |  |  | 0.8v |  | 1800 |
| 30 | VTH+12 |  |  |  | Should be greater than VTH-12 |  |